Active computational imaging for circumventing resolution limits at macroscopic scales

Prasanna Rangarajan,1,* Indranil Sinharoy,1 Predrag Milojkovic,2

Marc P Christensen1

1 Department of Electrical Engineering, Southern Methodist University, Dallas, Texas 75275-0338, USA

2 US Army Research Laboratory, RDRL-SEE-E, 2800 Powder Mill Road, Adelphi, Maryland 20783-1197, USA

*Corresponding author: prangara@smu.edu

Received XX Month XXXX; revised XX Month, XXXX; accepted XX Month XXXX; posted XX Month XXXX (Doc. ID XXXXX); published XX Month XXXX

Macroscopic imagers are subject to constraints imposed by the wave nature of light and the geometry of image formation. The former limits the resolving power while the latter results in a loss of absolute size and shape information. The suite of methods outlined in this work afford macroscopic imagers the unique ability to capture unresolved spatial detail while recovering topographic information. The common thread that binds these methods is the notion of imaging under patterned illumination. The notion is advanced further in developing computational imagers whose resolving power is decoupled from the constraints imposed by the collection optics and the image sensor. These imagers additionally feature support for multiscale reconstruction.

OCIS codes: (100.6640) Super-resolution; (110.1758) Computational Imaging; (150.0150) Machine vision; (100.2000) Digital image processing

http://dx.doi.org/10.1364/AO.99.099999

- INTRODUCTION

Since the invention of the microscope and the telescope, engineers and scientists alike, have embarked on a perpetual quest to improve the resolving power of imaging instruments.  Ernst Abbe was the first to recognize that the resolving power of an imaging instrument is fundamentally limited by diffraction.  Following Abbe's seminal work in 1873, the scientific community has made numerous attempts to circumvent the diffraction limit.  The most significant breakthroughs came in the form of Aperture Synthesis and Super-Resolution Microscopy, each of which has revolutionized imaging at the astronomical and microscopic scales, respectively.

Despite breakthroughs, there exists a continuum of image scales between the microscopic and the astronomical, where diffraction continues to limit the resolving power of imaging instruments. Existing approaches to improving the resolution of macroscopic imagers are centered on the notion of increasing the numerical aperture while concomitantly shrinking the pixel size. The former serves to improve the optical resolution by means of the diffraction limit, while the latter serves to improve the sensor resolution by means of the Nyquist limit. Attempts to break away from the endless cycle of creating better optics and smaller pixels are fraught with problems. These range from the poor light throughput of smaller pixels to the need for expertly designed optics featuring sophisticated aberration correction.

Recent innovations such as compressed sensing [1-3] permit us to break away from the Nyquist sampling limit but diffraction remains an unsurpassed barrier. The resulting loss of spatial detail is evidenced in the image of Figure 1 wherein the imager is unable to

Figure 1 	Illustrating the limits of macroscopic imaging

resolve the fine spatial detail in the inner segments of the spoke target.

A third limit that affects imaging at macroscopic scales stems from the loss of dimensionality (3D world→2D image) intrinsic to the geometry of image formation. It manifests as a loss of absolute size and shape information (perspective foreshortening), as evidenced in the image size of the chess pieces and the warped outline of the chessboard.

- Concept imager

The suite of methods outlined in this work afford macroscopic imagers the unique ability to circumvent the diffraction limit while also recovering depth/range information. Such disparate capabilities are realized by spatially patterning the light incident on the object volume prior to image acquisition. Subsequent processing of the images acquired under patterned illumination provides images with enhanced transverse and longitudinal resolution.

It is worth emphasizing that the idea of imaging under patterned illumination is not unique to the presented work. The microscopy community has exploited illumination diversity to circumvent the diffraction limit. Likewise, stereoscopic techniques in Computer Vision rely on illumination diversity to recover depth. An assimilation of these ideas into a broader framework for active computational imaging at macroscopic scales is one of the original contributions of this work.

Figure 2 provides a conceptual illustration of the proposed imager concept. It extends the notion of computational imaging to accommodate illumination diversity, thereby attempting to re-distribute the burden of imaging across the imaging, illumination, sensing and reconstruction components. The aforementioned notion contrasts the current practice of recording high fidelity imagery using

Figure 2 	Active Computational imaging concept

expertly designed optics and flood illumination.

The imager concept introduced above is a precursor to a new class of computational imagers dubbed "Active Computational Imagers". These imagers afford capabilities that are difficult to achieve using classical approaches. Examples include building computational imagers whose resolving power is fully decoupled from the constraints imposed by the collection optics and the sensor. Other features include support for multiscale reconstruction and ranging in a variety of mono-static and bi-static arrangements, and task specific engineering of the imager PSF [4].

The combined ability to range and super-resolve, while accommodating aberrations & aliasing, and support for multiscale reconstruction are capabilities that are unique to this work.

- Principle

The dual objectives of surpassing the diffraction limit and recovering depth information are realized by processing images acquired under spatially patterned illumination. The Moiré fringes arising from the heterodyning of object detail and patterned illumination encapsulate spatial frequencies lost to optical blurring. The deformations in the phase of the detected illumination pattern arising from parallax

Figure 3 	Mechanism underlying dual objectives of surpassing the diffraction limit and recovering depth information

encodes depth information.

- Surpassing the diffraction limit by heterodyning

The relevance of heterodyning to super-resolution is not apparent in the insets of Figure 3. The notion is explored further in the example of  Figure 4, wherein the imager is unable to resolve fine spatial detail in the target on account of the limited bandwidth of the imaging optics. Illuminating the target with a sinusoidal pattern produces sum and difference frequency patterns (Moiré fringes), the latter of which survives optical blurring. Knowledge of the frequency of the illumination pattern could be used to demodulate the fringes, yielding spatial frequencies beyond the optical cutoff. The demodulated image when combined with the diffraction limited image of the target yields a super resolved image whose bandwidth exceeds the diffraction limit. Closer examination of Figure 4 indicates that the target is modulated by multiple patterns that are phase-shifted replicas of a master pattern. The modulation diversity afforded by phase-shifting is used to restore the heterodyned spatial frequencies in an unambiguous

Figure 4 	Surpassing the diffraction limit by heterodyning

manner.

Lukosz & Marchand [5] were the first to recognize the potential use of Moiré fringes in extending the spatial frequency bandwidth of an imaging system. Their insight has spawned an extensive body of work [6-24] comprising active and passive techniques for super resolving extended objects. These techniques, however, restrict their attention to improving the resolution of well corrected optics characterized by space-invariant blur. The assumption of space-invariance contrasts the space-variance observed in practice (on account of defocus and/or aberrations). This work relaxes the space-invariance requirement and establishes that linearity of the imaging process is the only requirement for super resolution using heterodyning.

- Depth estimation using parallax

The depth map in Figure 5 is obtained by decoding the phase deformations in the recorded illumination pattern. It is observed that the deformations stem from the viewpoint difference between the

Figure 5 	Recovering depth using phase modulation

illumination source and the imager. The fact is routinely exploited in Laser Stripe Scanning [25-28], Structured Light Scanning [29-32] and Phase Measurement Profilometry [33-38].

Figure 6 Select experimental results highlighting specific capabilities of proposed active computational imagers.

Please note that in each example the super-resolved image contains spatial detail past the diffraction limit of the collection optics.

In general, the image of a scene acquired under patterned illumination exhibits both amplitude and phase modulation. The former is useful in recovering spatial detail lost to optical blur, while the latter is useful in recovering depth information. In brief, images acquired under spatially patterned illumination may be processed to recover an image with enhanced transverse and longitudinal resolution.

- Assumptions

The present work assumes the availability of incoherent light sources whose temporal fluctuations are largely uncorrelated over the time scale of integration. It is also assumed that the patterning of light is strictly confined to the irradiance component, and that the light sensitive elements in the illumination module are individually addressable as pixels. This assumption will be relaxed at a later time.

- Highlights

Figure 6 provides a snapshot of results compiled from various experiments at SMU. Details pertaining to each experiment are disclosed in upcoming sections.  The examples highlight the scope of the proposed approach, especially the latitude in selecting the optics and the sensor. In each instance, a distinct illumination pattern and reconstruction strategy is used to realize a specific task. In all instances, multiple images of the scene are acquired under patterned illumination. The images while monochrome in appearance, represent light integrated over the full spectral bandwidth of the detector.

The example on the top-left highlights the combined ability to super-resolve and recover depth, when using sinusoidal illumination patterns.

The example on the top-right relies on a stochastic illumination pattern (spread spectrum modulation) and a correlation receiver in surpassing the diffraction limit of a well-corrected optic observing multiple targets.

The example in the bottom-left demonstrates the ability to computationally engineer imagers whose resolving power is fully decoupled from the constraints imposed by the collection optics. The collection optics in this example is a single lens element exhibiting severe anisotropy (or space-variance) in resolving power. The results support the claim that space-variance is not a prerequisite for super resolution using heterodyning. The illumination/reconstruction strategy adopted in the example may be readily extended to accommodate detectors with varying degrees of aliasing (ranging from a single pixel to focal plane array with arbitrary pixel pitch).

The example in the bottom-right demonstrates the multiscale reconstruction capability afforded by the use of a lattice of light spots as the illumination pattern.

In summary, resolution gains in excess of 4× diffraction limit, and sub-mm range resolution over 1.1 m, were achieved under lab settings. In all cases, the resolution gain is limited by the resolving power of the illumination optics.

The discussions so far have presented “Active Computational Imaging” as a credible approach to circumventing resolution limits at macroscopic scale. The remainder of this work attempts to delve deeper into the mechanics of circumventing these limits.

Our inquiry begins with an attempt to develop a comprehensive model for irradiance transport in an active computational imager.

- MODEL FOR IMAGING UNDER PATTERNED ILLUMNATION

Current models for irradiance transport in structured light systems limit their attention to well corrected optics characterized by space-invariant blur. The subset of models concerned with macroscopic scales rely on a thin-lens approximation to describe the geometry of image formation. As a result, these models cannot accommodate the use of collection optics featuring multiple lens elements and apertures.

The present work outlines a general model for imaging under patterned illumination that simultaneously accommodates multi-lens systems and spatially varying optical blur.  The model draws upon established principles in optics and computer vision, including

- a pupil-centric thick-lens model for describing the geometry of image formation [39,40]

- the use of epipolar geometry to describe the geometric relation between corresponding points in the illumination and image fields [41,42]

- physical optics models for describing the spatially varying camera and projector optical blur [43].

The principal objective of this section is to identify the mathematical relation between the detector irradiance and the scene radiance originating from patterned illumination. In the interest of clarity, we initial efforts are restricted to the analysis of sinusoidal illumination patterns.

The notional active stereo arrangement of Figure 7 is used to introduce relevant concepts. The points Ocam & Oill represent the center-of-perspective (COP) of the imaging and illumination modules, respectively. It is assumed without loss of generality that the world coordinate system (XYZ) is aligned with the camera coordinate system centered at Ocam.

Subsequent discussions use the terms camera and imager interchangeably. Likewise, the terms projector and illumination source

Figure 7 	Notional active stereo arrangement used to develop model for imaging under patterned illumination

are used interchangeably.

In Figure 7, the points cx,cy & (cx,cy) represent the non-zero pixel coordinates corresponding to the intersection of the camera and projector optical axes with the respective sensor and display planes. The vector OcamOill joining the camera and projector COP is referred to as the baseline vector or baseline, in keeping with nomenclature in computer vision.

The mathematical expressions developed in this section will oftentimes simultaneously accommodate spatial and spatial-frequency coordinates. To minimize confusion, a standard notation has been adopted. For example, accented/unaccented coordinates denote points in the projector/camera image field respectively. A more exhaustive list is compiled below:

| 3D Coordinates | Coordinates of scene point e.g.: X,Y,Z) |  |
| --- | --- | --- |
| Unaccented 2D Coordinates | Pixel coordinates of field point in image sensor e.g.: x,y |  |
| Accented 2D Coordinates | Pixel coordinates of field point in display e.g.: x,y |  |
| Uppercase Boldface | Matrices | Mij : element in ith row and jth column |
| Lowercase Boldface | Column vectors |  |
| F{…} | Fourier transform operator |  |
| Caligraphic Typeface | Fourier transform of a spatial pattern or 2D function |  |
| Greek symbols ξ,η | Spatial frequency coordinates expressed in cyclesmm or cyclespixel |  |
| b | L2 norm of vector |  |

- Geometric Model

Suppose that the imager in Figure 7 observes the scene point P≝(X0,Y0,Z0), as illustrated in Figure 8.

Figure 8 	Illustration of imaging model for a canonical collection optic

The relation between the scene point and its image coordinates p≝x0,y0 is disclosed in Eq.(1). The negative sign in the expressions accommodate the image reversal observed at the sensor.

| x0= -mpΔZdZoX0 +cx | (1) |
| --- | --- |
| y0= -mpΔZdZ0Y0 +cy |  |

It is worth emphasizing that the values associated with x0& y0 are expressed in pixel units and do not represent field coordinates. This slight abuse of notation allows us to readily incorporate standard results from epipolar geometry in computer vision.

The pixel offsets cx,cy In Eq. (1) accommodate the fact that the origin of the sensor pixel coordinate system is located at the top-left corner of the sensor modules. The term Δ in represents the pixel pitch of the image sensor, while the term Zd represents the distance between the camera exit pupil plane and the image sensor.

The central projection relation disclosed in Eq.(1) is derived using the pupil-centric model for image formation documented in literature [39-40]. It assumes that the lens elements and apertures may be aggregated into a single unit comprised of two terminals: an “entrance pupil” representing the finite aperture through which light-rays enters the optical elements, and an “exit pupil” representing the finite aperture through which light-rays exit the optical elements en-route to the sensor. The passage of light through the unit is described using the standard paraxial model for thick-lenses.

In view of the above discussion, the COP of the illumination and imaging modules points Ocam & Oill are seen to correspond to the center of the entrance and exit pupil planes of the imaging and illumination modules, respectively.

The object-image relation disclosed in Eq.(1) is obtained by observing that the direction cosines of the incoming segment of the chief ray ocamp and the outgoing segment OcamP are related through the pupil magnification mp. In the simpler case of unity pupil magnification, the chief ray angles on the object and image side are identical, and the resulting expressions reduce to the imaging model frequently encountered in computer vision.

Notice that object-side and image-side distances are specified with reference to the entrance and exit pupil’s respectively.

The relation disclosed in Eq.(1) can be re-formulated as the matrix product shown below:

| x0y01=1Z0-mpZd/Δ 0cx0-mpZd/Δcy001KX0Y0Z0 | (2) |
| --- | --- |

Readers familiar with computer vision will recognize the matrix K as the camera intrinsic matrix.

The imaging model disclosed above may also be used to identify the relation between the scene point (X0,Y0,Z0) and its pixel counterpart x0,y0 in the projector field. The result is expressed as the matrix product shown below:

| x0y01=γ-mpZd/Δ0cx0-mpZd/Δcy001Kr11r12r13r21r22r23r31r32r33RX0-bXY0-bYZ0-bZ | (3) |
| --- | --- |
| where γ=1r31X0-bX+r32Y0-bY+r33Z0-bZ |  |

The term Δ represents the pixel pitch of the projector, while the term Zd represents the distance between the projector entrance pupil plane and the light sensitive module.

The relation disclosed in Eq.(3) is derived by transforming the world coordinates of the scene point into an intermediate Cartesian coordinate frame centered at Oill, such that its Z-axis is aligned with the projector optical axis. The vector X0-bX,Y0-bY,Z0-bZT accommodates the translation of the origin of the intermediate coordinate frame w.r.t the world coordinate frame. Multiplication by R accommodates the difference in the orientation of the world coordinate frame and the intermediate coordinate system centered at Oill.

Readers familiar with computer vision will recognize the matrix K as the projector intrinsic matrix.

- Mapping from projector ↔ camera pixel coordinates

Using the object-image and display-image relations disclosed in Eqs.(4)-(5) one can now attempt to identify the geometric mapping between the camera and projector pixel coordinates. We begin by eliminating the vector X0,Y0,Z0T from Eqs.(4)-(5) The resulting vector identities are disclosed below,

| x0y01=γZ0KRK-1x0y01- γKRK-1KbXbYbZ | (4) |
| --- | --- |
| x0y01=1γ1Z0KRTK-1x0y01+1Z0KbXbYbZ | (5) |

The non-singular matrices KRK-1, KRTK-1 define a bi-linear mapping (homography / projective transform) between the pixel coordinates of the camera and projector, for objects at infinity (Z0→∞). These matrices are referred to as the infinite homography in vision literature. The vector KbX,bY,bZT represents the geometric image of the projector’s center-of-perspective as seen from the camera viewpoint, and is referred to as epipole in vision literature.

In the interest of notational brevity, the matrices KRK-1, KRTK-1 and the vector KbX,bY,bZT will henceforth be abbreviated as H∞, H∞ and t respectively. The abbreviations allow us to express Eqs.(4)-(5) in the following compact form:

| x0y01=γZ0H∞ x0y01-γH∞tXtYtZ | (6) |
| --- | --- |
| x0y01=1γ1Z0H∞x0y01+1Z0tXtYtZ | (7) |

It is worth emphasizing that the infinite homography matrices H∞, H∞and the epipole t are completely independent of scene geometry and depend exclusively on the arrangement of the camera and projector.

Equating the third element on either side of Eq.(6) allows us to solve for γ. Substituting the resulting expression for γ back into Eq.(6), yields the following camera → projector coordinate mapping:

| x0=h11∞(Z0x0-tX+h12∞(Z0y0-tY+h13∞(Z0-tZh31∞(Z0x0-tX+h32∞(Z0y0-tY+h33∞(Z0-tZ | (8) |
| --- | --- |
| y0=h21∞(Z0x0-tX+h22∞(Z0y0-tY+h23∞(Z0-tZh31∞(Z0x0-tX+h32∞(Z0y0-tY+h33∞(Z0-tZ |  |

Alternately, one can solve for γ by equating the third element on either side of the vector-identity of Eq.(7). Substituting the resulting expression for γ, yields the following projector → camera coordinate mapping:

| x0=Z0-tZZ0h11∞x0+h12∞y0+h13∞h31∞x0+h32∞y0+h33∞+1Z0tX | (9) |
| --- | --- |
| y0=Z0-tZZ0h21∞x0+h22∞y0+h23∞h31∞x0+h32∞y0+h33∞+1Z0tY |  |

The expression for the projector ↔ camera coordinate mapping disclosed in Eqs.(8) & (9) are essential to the process of transporting irradiance from the projector to the scene and on to the camera. Variants of the expression were originally identified by the authors in [44]. A more detailed derivation of the current expression for the projector ↔ camera coordinate mapping is available in [4].

- Limitations

The imaging model disclosed in Eqs.(2)-(3) assumes central projection consistent with the assumptions of paraxial imaging. Any deviations from this behavior can be accommodated as a source of space-variance in the optical blur. As an example radial and pincushion distortion could be modelled as a source of field-dependent aberrations through the  W311 Seidel aberration term.

- Blur Model

Accurate modelling of the light transport from the projector to the camera requires simultaneous consideration of the projector ↔ camera coordinate mapping, and the blurring induced by the imaging and illumination optics. Blurring on the image side limits the ability to resolve increasingly fine detail, while blurring on the illumination side limits the ability to project high frequency spatial patterns.

The remainder of this section is devoted to assimilating the geometric model developed in the previous section with physical optics models for describing optical blur in the imaging and illumination paths.

- Light transport from the scene to the detector

Our inquiry begins with an attempt to transport light from the scene to the detector. To this end, we interpret the illuminated object volume as a collection of secondary point emitters/sources. The optical image of each point-source is a blurry spot commonly referred to as the point-spread function (PSF). The PSF neatly encapsulates the effect of blurring due to diffraction, optical aberrations and defocus. The spatial extent of the PSF imposes a firm limit on the resolving power of the imaging system.

Suppose that the camera in Figure 7 observes a point source located at U0,V0,W0 in the object space. Suppose that  (u0,v0) represent the transverse coordinates of the Gaussian image of the scene point (U0,V0,W0). A real-valued function of the form hcam(x-u0,y-v0;u0,v0) encapsulates the light distribution at x,yth detector pixel, in response to the point source at (U0,V0,W0). The arguments (u0,v0) in hcam(x-u0,y-v0;u0,v0) capture the field dependence of the PSF w.r.t the location of the point source. Lohmann & Paris [43] extended the above notion to an arbitrary collection of point sources using the super-position integral, shown below The result is summarized below

| ix,y=p(u,v) hcam(x-u,y-v;u,v)du dv | (10) |
| --- | --- |

The term p(u,v) represents the intensity associated with a point source at U,V,W, whose Gaussian image coordinates are (u,v).

Interpreting the illuminated object volume as a collection of secondary point sources, allows us to use Eq.(10) as a blueprint for identifying the expression for the camera image iθx,y. The corresponding expression for iθx,y is included below:

| iθx,y=su,v×ru,v hcam(x-u,y-v;u,v)du dv | (11) |
| --- | --- |

The term su,v represents the camera image of the illumination pattern, while ru,v represents the geometric image of the scene under uniform illumination.

- Light transport from the projector to the scene

Without loss of generality, the expression for the detected intensity disclosed in Eq.(11), may be adapted to obtain the expression for the radiance at the scene point (X,Y,Z) arising from the illumination pattern pθx,y. The corresponding expression is provided below:

| sX,Y,Z∝pθu,vhill(x-u,y-v;u,v) dudv | (12) |
| --- | --- |

The term pθu,v represents the intensity of intensity of the u,vth pixel in the illumination pattern, while hill(x,y;u,v) represents the spatially varying blur associated with the illumination optics.

- Light transport from the projector to the camera

The expressions for the incident and detected intensities disclosed in Eqs.(11)-(12) provide a blueprint for the design of a comprehensive model for light transport in the active stereo arrangement of Figure 7.

Subsequent discussions on the topic restrict their attention to the following sinusoidal patterns:

| Periodic sinusoidal pattern pθx,y=A+Bsin2πξ0x+η0y+θ | (13) |
| --- | --- |
| Warped sinusoidal pattern pθx,y=A+Bsin2πξ0h11∞x+h12∞y+h13∞h31∞x+h32∞y+h33∞+η0h21∞x+h22∞y+h23∞h31∞x+h32∞y+h33∞+θ | (14) |

The terms ξ0,η0 represent the spatial frequency of the illumination pattern, as expressed in cycles/pixel. The terms A, B represent the average intensity and peak intensity excursion of the illumination pattern. The term θ represents the phase shift associated with the illumination pattern. The terms hij∞ represent the i,jthentry of the infinite homography H∞=KRTK-1.

The periodic sinusoidal pattern of Eq.(13) is the pattern of choice in Super-Resolution Microscopy and Phase Measurement Profilometry. The warped sinusoidal pattern of Eq.(14) is the outcome of a joint effort to recover scene geometry and spatial detail lost to blurring. Its use is unique to this work.

Light transport under periodic sinusoidal illumination

The transport model of Eq. (12) may be used to may be used to identify the incident intensity at the scene point X,Y,Z, in response to the periodic sinusoidal pattern of Eq.(13). The result is provided below:

| sX,Y,Z=Ax,y+Bx,ysin2πξ0x+η0y+∠Bu,v+θ | (15) |
| --- | --- |

The terms Ax,y, Bx,y and ∠Bx,y encapsulate the effect of the illumination optics on the contrast of the projected pattern, and the instantaneous phase of the projected pattern.

A fraction of the light incident on X,Y,Z reaches the x,yth camera pixel, following albedo loss and optical blur. The expression for the intensity of the x,yth pixel is obtained by aggregating the contribution of every scene point U,V,W to the x,yth pixel, just as in Eq.(11). The resulting expression for the detector irradiance is disclosed below

| iθx,y=ru,vAu,v+Bu,vsin2πξ0u+η0v+∠Bu,v+θhcamx-u,y-v;u,v dudv | (16) |
| --- | --- |

The term ru,v in Eq.(16) represents the geometric image of the scene obtained under uniform illumination.

The coordinate mapping x,y⟼x,y identified in Eq.(8) may be used to recast the expression for the detector irradiance in the form:

| iθx,y=ru,vAu,v+Bu,v sinφu,v+θ+∠Bu,vhcam(x-u,y-v;u,v)dudv | (17) |
| --- | --- |

The definition of the various terms in Eq.(17) is included below

| φu,v≝2πξ0h11∞Wu-tX+h12∞Wv-tY+h13∞W-tZh31∞Wu-tX+h32∞Wv-tY+h33∞W-tZ +2πη0h21∞Wu-tX+h22∞Wv-tY+h23∞W-tZh31∞Wu-tX+h32∞Wv-tY+h33∞W-tZ |  | (18) |
| --- | --- | --- |
| Au,v≝Ahillu-u,v-v;u,vdudv |  |  |
| Bu,v≝exp-j2πξ0u-u+η0v-v×hillu-u,v-v;u,vdudv |  |  |

The term φu,v represents the parallax induced phase distortion experienced by the illumination pattern, when observed from the camera perspective. Likewise, the terms Ax,y, Bx,y and ∠Bx,y represent the blur induced distortion in the contrast and phase of the detected illumination pattern.

A comprehensive derivation of Eqs.(15) & (17) is available in Section 3.4 of [4].

Light transport under warped sinusoidal illumination

The transport model of Eq. (12) may be used to may be used to identify the incident intensity at the scene point X,Y,Z, in response to the warped sinusoidal pattern of Eq.(13). The resulting expression is identical to Eq. (15).

A fraction of the light incident on X,Y,Z reaches the x,yth camera pixel, following albedo loss and optical blur. The expression for the intensity of the x,yth pixel is obtained by aggregating the contribution of every scene point U,V,W to the x,yth pixel, just as in Eq.(11). The resulting expression for the detector irradiance is disclosed below

| iθx,y=ru,vAu,v+Bu,v sinφu,v+θ+∠Bu,vhcam(x-u,y-v;u,v)dudv | (19) |
| --- | --- |

The definition of the various terms in Eq.(19) is included below

| φu,v≝2πW-tZWξ0u+η0v-2πW-tZξ0tX+η0tY |  | (20) |
| --- | --- | --- |
| Au,v≝Ahillu-u,v-v;u,vdudv |  |  |
| Bu,v≝exp-jφu-u,v-v×hillu-u,v-v;u,vdudvdudv |  |  |

The derivation employs the coordinate mapping x,y⟼x,y identified in Eq.(9). A comprehensive derivation of Eqs.(19)& (20) is available in Section 3.5 of [4].

The term φu,v in the expression for the detector irradiance (Eq.(19)) represents the parallax induced phase distortion experienced by the illumination pattern, when observed from the camera perspective.

Likewise, the terms Ax,y, Bx,y and ∠Bx,y represent the blur induced distortion in the contrast and phase of the detected illumination pattern.

- Highlights

Eqs.(17)-(20) furnish a rigorous model for imaging under sinusoidal illumination that simultaneously accommodates space-variant blur in the imaging/illumination paths, and topographic variation in the scene.

On account of the linearity of the light transport mechanism, these expressions for the detector irradiance may be readily extended to accommodate illumination patterns that are expressible as a superposition of sinusoids or warped sinusoids.

- Key findings

Figure 9 	Expression for detector irradiance in an active stereo setup

Figure 9 succinctly captures two key result that emerges from the analysis of light transport in structured light systems.

Notice that the camera perceives a scene-dependent amplitude and phase variation in the projected pattern. The finding is consistent with practical observations.

Furthermore, the camera image is a blurred representation of the ideal geometric image of the scene, subject to amplitude and phase modulation by a pattern whose functional form is identical to the functional form of the illumination pattern.

Super-resolution by heterodyning is predicated on the observation of unresolved spatial frequencies in the camera images acquired under sinusoidal illumination. The presence of such frequencies may be confirmed by analyzing the Fourier transform of the product term  ru,vAu,v+Bu,v sinφu,v+∠Bu,v+θ. The field-dependent amplitude distortions arising from Au,v and B(u,v) diminish the modulation strength of the projected pattern, limiting the ability to heterodyne increasingly fine object detail.

Depth estimation is predicated on the observation of scene dependent phase deformations in the camera images of the projected illumination pattern. The scene-dependent phase modulation term φ(u,v) serves this purpose, and permits recovery of depth information W from φ(u,v), through Eqs.(18)&(20). The additional phase distortion ∠Bu,v attributed to the illumination blur, results in an over/under estimation of the scene depth W. The field-dependent amplitude distortions arising from Au,v and B(u,v) diminish the modulation strength of the projected pattern, resulting in a loss of range resolution. These issues are examined in detail in [4].

- Limitations

The light transport model developed in this work disregards cast shadows and irradiance contributions from indirect light paths involving inter-reflections and/or scattering in the scene. Such behavior is expected to introduce bias in the recovered scene geometry and artifacts in the super-resolved image. A rigorous treatment of the topic is beyond the scope of this paper.

- Useful restrictions

It is evident from the discussion in Section-2(B)5 that neither space-variance in the optical blur, nor topographic variations in a macroscopic scene, preclude the possibility of super-resolution using patterned illumination.

At first glance, it appears that the heterodyned frequencies can be restored to their correct position outside the optical passband, using an AM (amplitude modulation) receiver that is tuned to the spatial frequency ξ0,η0 of the illumination pattern pθx,y. In practice it is observed, exact restoration is only supported by select camera + projector arrangements. The inconsistency stems from the fact that the detected sinusoidal pattern exhibits scene-dependent phase distortion i.e., φ(u,v) depends on scene depth W. Such distortions are evident in the image of the chess pieces in Figure 10, and at the interface of the vertical planes and the ground plane. The scene-dependence while desirable from a depth estimation standpoint, presents problems for super-resolution. Failure to compensate for these scene-dependent distortions produces

Figure 10 	Camera image of scene under periodic sinusoidal illumination in the active stereo arrangement of Figure 7

aliasing like artifacts in the demodulated/super-resolved image.

A formal discussion of the mechanics of super-resolution and depth estimation will be deferred until Section-3. The remainder of this section is devoted to the study of combinations of stereo arrangements and illumination patterns, for which the camera image of the projected pattern is devoid of scene-dependent phase distortions, i.e.,  φ(u,v) is independent of scene depth.

- Monostatic arrangement for periodic sinusoidal illumination

The expressions for the detector irradiance iθx,y (listed in Eq.(17)) and the phase φ(u,v) (listed in Eq.(18)) provide clues to eliminating scene-dependent distortions when using periodic sinusoidal patterns.

It follows from Eq.(18) that φ(u,v) exhibits a bi-linear dependence with the scene depth W. This bi-linear dependence may be reduced to a strict inverse dependence, under a specific choice of infinite homography H∞, epipole t and pattern orientation ξ0,η0. The resulting expression for φ(u,v) is disclosed below:

| φu,v=2πκoξ0u+η0v+φo+1W2πκdξ0bX+η0bY | (21) |
| --- | --- |
| wherein κo≝mpmpZdZdΔΔ, κd≝mpZdΔ, φo≝2πξ0(cx-κocx)η0(cy-κocy)+ |  |

The term κo accommodates differences in the transverse magnification of the illumination and imaging paths, while φo accommodates differences in the sampling phase of the sensor and display pixel grids.

Constraints on stereo arrangement

- infinite homography H∞ reduces to an affine transform, i.e. h31∞=h32∞=0 and h33∞=1

- Z-component of the epipole tz=0

These constraints are simultaneously satisfied when the camera and projector optical axes are parallel and the baseline vector is perpendicular to both optical axes, i.e., R=I and bZ=0). This stereo arrangement is referred to as the canonical stereo arrangement in vision literature. In such arrangements the entrance pupil plane of the camera and the exit pupil plane of the projector are coplanar, so that there is no Z-displacement between their COP’s. The net result is that the camera and projector side epipoles are both at infinity.

Constraints on pattern orientation

The phase function φ(u,v) simplifies to a pure carrier frequency favored by super-resolution when the pattern orientation satisfies the constraint ξ0bX+η0bY=0. In such cases, the iso-intensity lines in the illumination pattern are aligned with the camera-side epipolar lines [46,47]. Deviations from the above choice produce depth-dependent phase distortions useful for depth estimation. Both features were observed and verified in experiments. Figure 11 illustrates these features for a canonical stereo arrangement with a

Figure 11 	Canonical active stereo arrangement with epipoles at infinity

vertical baseline.

The discussion thus far seems to suggest that the ability to super-resolve is confined to a single orientation. The problem is easily addressed by either employing multiple canonical stereo arrangements sharing a common imager, or rotating a single canonical stereo arrangement about the camera COP. Additional details are available in [4].

The chief limitation of the canonical stereo arrangement is the inability to accommodate a large baseline that is necessary to improve the depth resolution of the computational imager. The above fact is evident in the linear/inverse dependence of the phase φu,v with the baseline/scene-depth in Eq.(21). Attempts at increasing the baseline in canonical stereo arrangements are likely to fail on account of the limited overlap between the illuminated volume and the imaged volume.

- Monostatic arrangement for warped sinusoidal illumination

The expressions for the detector irradiance iθx,y (listed in Eq. (19)) and the phase φ(u,v) (listed in Eq. (20)) provide clues to eliminating scene-dependent distortions when using periodic sinusoidal patterns.

It follows from Eq. (20) that φ(u,v) exhibits a bi-linear dependence with the scene depth W. This bi-linear dependence may be reduced to a strict inverse dependence, under a specific choice of the epipole t and pattern orientation ξ0,η0. The resulting expression for φ(u,v) is disclosed below:

| φu,v=2πκoξ0u+η0v+φo+1W2πκdξ0bX+η0bY | (22) |
| --- | --- |
| wherein κo≝1, κd≝1, φo≝0 |  |

Constraints on stereo arrangement

- Z-component of the epipole tz=0

This constraint is satisfied when the projector COP lies in the entrance pupil plane of the camera. The net result is that the camera side epipole is at infinity. Such stereo arrangements are referred to as collocated stereo arrangements in this work.

Constraints on pattern orientation

The phase function φ(u,v) simplifies to a pure carrier frequency favored by super-resolution when the pattern orientation satisfies the constraint ξ0bX+η0bY=0. In such cases, the iso-intensity lines in the illumination pattern are aligned with the camera-side epipolar lines [44]. Deviations from the above choice produce depth-dependent phase distortions useful for depth estimation.

Both features were observed and verified in experiments. Figure 11 illustrates these features for a canonical stereo arrangement with a

Figure 12 	Collocated stereo arrangement with camera epipole at ∞

horizontal baseline.

The principal difference between the canonical and the collocated stereo arrangements lies in the non-coplanar structure of the pupil planes and the non-zero angle between the optical axes. The warped nature of the sinusoidal illumination pattern accounts for the relative rotation between the camera and projector optical axes. As a result, the warped sinusoidal illumination pattern appears as a periodic sinusoid to the camera. In the absence of the warping a pure sinusoidal pattern would appear to the camera as a chirp pattern. The supplementary material [45] includes a video that illustrates the benefit of warping.

Broader impact

It is observed that the stereo arrangements in commodity structured light (SL) scanners tend to operate in near-collocated configurations. This suggests that commodity SL scanners can be adapted (with minimal modification) to super-resolve texture in addition to recovering topographic information. This unique capability has hitherto remained undocumented in the optics and vision communities.

Lastly, the canonical stereo arrangement of Section 2 may be interpreted as a special case of the collocated stereo arrangement, so that one could attempt to pre-warp the periodic sinusoidal illumination pattern in a canonical stereo arrangement. In this case, pre-warping amounts to compensating for the difference in the relative magnification of the camera and projector, and also the difference in the sampling phase of the camera and projector sampling grids.

- Monostatic arrangement for any illumination pattern

Attempts to super-resolve in multiple orientations using the canonical and collocated stereo configurations either requires multiple projectors or it requires mechanical movement. These issues may be overcome by examining stereo arrangements for which the phase function φ(u,v) (listed in Eq.(18)) is independent of scene depth. Direct substitution can be used to verify that when the camera and projector share the same viewpoint R=identity matrix,bX=bY=bZ=0, the phase function φ(u,v) is completely independent of scene depth. The resulting expression for φ(u,v) is disclosed below:

| φu,v=2πκoξ0u+η0v+φo | (23) |
| --- | --- |
| wherein κo≝mpmpZdZdΔΔ, φo≝2πξ0(cx-κocx)η0(cy-κocy)+ |  |

A 45∘ beam splitter is typically used to ensure that the camera and projector share a common viewpoint. It is essential that the COP’s of the camera and projector are lined up, in addition to their optical axes. The aforementioned stereo arrangement is referred to as coincident stereo in view of the fact that the two COP’s coincide with each other.

An attractive feature of the coincident stereo arrangement is that any illumination pattern (sinusoidal or otherwise) appears devoid of phase deviations from the camera perspective. For this reason, it is the preferred embodiment for realizing super-resolution using any spatial pattern.

Intuition suggests that the absence of scene-dependent phase distortions in the phase function φ(u,v) would impede attempts to recover topographic information in a coincident stereo arrangement. In such cases, the finite depth of field of the projector may be exploited in recovering depth information from the modulation strength given by Au,vBu,v of the detected pattern [48].

- Bistatic arrangement for periodic sinusoidal pattern and combinations thereof

The canonical/collocated active stereo arrangements impose specific restrictions on the placement (bX,bY,bZ) of the camera and projector. The common restriction bZ=0 appears to automatically rule out bistatic arrangements. Further analysis reveals that these restrictions may be lifted when attempting to super-resolve a single planar facet that is plane-parallel to both the camera and projector pupil planes. In such cases, the relative rotation R=identity matrix, and the absolute depth of each scene point reduces to a constant W0 that is independent of the field coordinates (u,v). Incorporating the aforementioned values into Eq.(18) yields the following expression for phase function φu,v:

| φu,v=2πκoξ0u+η0v+φo | (24) |
| --- | --- |
| wherein κo≝mpmpZdZdΔΔW0W0-tZ and φo≝2πξ0cx-mpmpZdZdΔΔcx+2πη0cy-mpmpZdZdΔΔcy-2πκoW0 ξ0tX+η0tY |  |

Notice that the linear phase profile of φu,v is consistent with that desired by super-resolution.

In subsequent discussions, the term 2D scene is used to describe the planar facet that is plane-parallel to both the camera and projector pupil planes. The nomenclature reflects the fact that these scenes do not exhibit no topographic variation from the camera perspective.

A bistatic active stereo arrangement was used to super-resolve the 2D scenes in Panels 2-4 of Figure 6.

- SUPER-RESOLUTION USING SINUSOIDAL PATTERNS

As stated previously, active super-resolution is predicated on the prospect of observing unresolved spatial frequencies in the camera images acquired under sinusoidal illumination. It was established in Section-2(C) that a select combination of stereo arrangements and sinusoidal patterns support pure amplitude modulation by a carrier, as favored by super-resolution. However, it remains to be established that the modulated spatial frequencies can be unambiguously restored, effecting an improvement in the resolving power/bandwidth of the collection optics.

We outline a demodulation strategy utilizing standard trigonometric identities to process the images acquired under sinusoidal illumination.

- Demodulation Strategy

The demodulation strategy adopted in this work is encapsulated in the expression for the reconstructed image, disclosed below

| ireconx,y≝ ibbx,y+ cos2πκoξ0x+η0y+φ0 icosx,y+sin2πκoξ0x+η0y+φ0 isinx,y | (25) |
| --- | --- |

The term ibb(x,y) dubbed the “baseband image”, represents the camera image acquired under uniform illumination. The terms icosx,y & isinx,y represent images that are acquired under strictly cosine & sine illumination respectively. The scale factor κo accommodates differences in the transverse magnification of the illumination and imaging paths. The phase offset φo accommodates differences in the sampling phase of the detector pixel grid and the projector pixel grid.

The images ibb, icos , isin are derived from the set of camera images iθx,y acquired under the phase shifted illumination pattern

pθx,y : θ∈0,π2,π,3π2, using Eqs.(26)– (28).

| ibbx,y≝14i0x,y+iπ/2x,y+iπx,y+i3π/2x,y | (26) |
| --- | --- |
| icosx,y≝12iπ/2x,y-i3π/2(x,y) | (27) |
| isinx,y≝12i0x,y-iπ(x,y) | (28) |

The expression for the reconstructed image ireconx,y may be simplified by utilizing the trigonometric identity cosP-Q=cosPcosQ+sinPsinQ. The simplified expression for the reconstructed image ireconx,y is disclosed below:

| ireconx,y=ru,vo(x-u,y-v;u,v)hcamx-u,y-v;u,v dudv | (29) |
| --- | --- |
| wherein o(x,y;u,v)≝Au,v+Bu,vcos2πκoξ0x+η0y-∠Bu,v | (30) |

Notice that the reconstructed image irecon(x,y) bears a strong resemblance to the image acquired under the computationally engineered PSF,

| hengdx,y;u,v≝ox,y;u,v hcamx,y;u,v. | (31) |
| --- | --- |

- Frequency domain interpretation of super-resolution

The terms Au,v,Bu,v and  hcamx,y;u,v embedded within the expression for the engineered PSF remain fixed values for a fixed field location u,v. The deterministic nature of these terms allow us to associate a field dependent transfer function to the imager and the computationally engineered imager. The standard practice of identifying the transfer function by computing the Fourier transform of the PSF can be utilized to obtain the following result:

| Fhengdx,y;u,v=Fox,y;u,v⊛Fhcamx,y;u,v | (32) |
| --- | --- |

The operator F denotes the 2D Fourier transform, while ⊛ denotes the convolution operator.

It is observed that the convolution operation in Eq.(32) produces replicas of the field-dependent camera OTF, which are centered at the frequencies 0,0 & ±κoξ0Δ,κoη0Δcyclesmm. The replication in frequency domain serves to increase the bandwidth of the computationally engineered OTF and the underlying computational imager.

- Spatial domain interpretation of super-resolution

Intuition suggests that an increase in the bandwidth of the computationally engineered OTF is likely accompanied by a reduction in the size of the resolvable spot. An accepted criterion for identifying the size of the resolvable spot is the width of the central lobe. It is identified as the separation of the intensity minima lying on either side of the intensity maximum. The multiplicative structure of Eq.(31) guarantees that the engineered PSF shares the minima of the camera PSF and the oscillatory pattern. In theory, a tightening of the central lobe should result, when two or more minima of the oscillatory pattern ox,y;u,v are accommodated within the central lobe of the camera PSF hcam(x,y;u,v). We proceed to verify this claim using an example.

The field dependence of the terms Au,v,Bu,v complicates the interpretation of PSF engineering. So we assume (strictly for the purpose of this discussion) that the illumination blur is space-invariant, i.e., Au,v=A,Bu,v=B, and κo=1.

Figure 13 illustrates the prospect of super resolving a diffraction limited imager, using periodic sinusoidal illumination. The plots labeled “Optical PSF” represent cross-sections of an Airy disk with cutoff frequency ρ0=193.69 cyc/mm. The red lines depict the spacing between the first two nulls. The plots labeled “Engineered PSF” represent cross-sections of the engineered PSF in two principal directions: one aligned with the modulation direction and the other orthogonal to the modulation direction. A modulation frequency of ξ0=0,η0=ρ0=193.69 cyc/mm was chosen for the purpose of this discussion. Notice that the resolution gain in the modulation direction manifests as a reduction in the central lobe width of the engineered PSF. The finding is consistent with the claims made in previous paragraphs. It is observed that 4 consecutive intensity minima of the oscillatory pattern can be accommodated within the central lobe of the Airy disk. Each intensity minimum marks the beginning of a side lobe in the engineered PSF.

Figure 13 	Super-resolution of a diffraction limited imager

effected by computational PSF engineering

Intuition suggests that a larger resolution gain should result when multiple intensity minima of the oscillatory pattern are accommodated within the central lobe of the Airy disk. In practice it is observed that the side lobes manifest as ghost artifacts in the reconstructed image. The problem stems from the highly repetitive structure of the sinusoidal illumination pattern, and the associated oscillatory pattern.

Figure 14  illustrates the prospect of engineering the PSF of a 19mm biconvex lens using a periodic sinusoidal pattern with frequency  ξ0=0,η0=125 cyc/mm. The diffraction limited cutoff frequency of the singlet is  ρ0=193.69  cyc/mm.

Figure 14 Super-resolution of a 19mm biconvex lens

effected by computational PSF engineering

Inspection of the engineered PSF’s confirms resolution improvement in the modulation direction. Closer inspection of the off-axis engineered PSF’s reveals pronounced side-lobes. The presence of these lobes is attributed to the increased number of cycles of the oscillatory pattern o(x,y;u,v) that can be accommodated within the central lobe of the off-axis PSF’s.

The examples discussed thus far support the notion that super resolution is effected by computational engineering of the transverse PSF. The notion generalizes the viewpoint espoused in [49] for space-invariant blurs, and is an original contribution of this work.

- Aliasing management

The analysis of super-resolution thus far has implicitly assumed that the detector Nyquist limit exceeds the diffraction limited cutoff frequency of the imaging optics. The assumption may be violated in practice. Our super-resolution technique accommodates mild undersampling (e.g.: Nyquist-frequency < 0.75 × optical-cutoff) by digitally low-pass filtering (DLPF) the recorded images prior to demodulation. The cutoff frequency of the DLPF is chosen to block all aliased frequencies (>0.25 × optical-cutoff in example) in the recorded image. The additional loss in resolution stemming from DLPF is overcome by using additional sinusoidal illumination patterns.

Severe undersampling in the recorded images introduces irreversible ambiguities in the position of the heterodyned frequencies. The issue is overcome in Section-5 by adopting a different modulation (illumination pattern) and reconstruction strategy.

Even in the absence of any undersampling, the finite pixel pitch of the detector could present problems during demodulation. A subset of the demodulated frequencies may exceed the sensor Nyquist frequency producing aliasing artifacts in the reconstructed image ireconx,y. This issue was observed and reported by the authors in [50]. It was observed that resampling the images ibbx,y, icosx,y and  isin(x,y) resolves the problem. Spatial domain interpolation techniques such as Lanczos interpolation may be employed for the purpose. The results in Panel-1 of Figure 6 were produced with appropriate resampling prior to demodulation.

- Experimental validation

In the interest of brevity, we examine two experiments with the objective of corroborating the findings of this work, and demonstrating the feasibility of super-resolution by heterodyning. A more exhaustive list of experiments validating the proposed approach in a variety of monostatic and bistatic arrangements is compiled in [4, 44] and [47].

- Super-resolving well corrected optics

Figure 15 	Super-resolution in a collocated active stereo arrangement

Details of the apparatus used to demonstrate resolution improvement of a well-corrected optic observing a macroscopic scene with topographic variations is included in Figure 15.

Results from the experiment are included in panel-1 of Figure 6. We briefly outline the procedure used to set up the collocated stereo arrangement and identify the warped sinusoidal pattern.

The three optical posts of increasing height embedded within the yellow rectangle of Figure 15 are used to align the camera optical axis with a line on the optical table, and to ascertain that the camera rotates about this line through the COP. A thin and sharp tipped object such as the lead of a mechanical pencil is used to ascertain that the projector COP is contained within the camera entrance pupil plane. Collocation in the horizontal direction is achieved when the tip of the lead and its shadow under flood illumination from the projector are lined up horizontally. Details will be made available in the supplementary material.

The warped sinusoidal pattern required for super-resolution is identified using Eq.(14). The process requires knowledge of the infinite homography H∞, whose computation is tedious. In previous work [44], we established that the homography Π induced by a planar facet that is plane-parallel to the projector exit pupil may be used to identify the warped sinusoidal pattern warp in lieu of the infinite homography. The facet labelled π in Figure 15 serves this purpose.

The homography is computed using techniques from vision [51] that rely on matching corresponding features in the camera image of a reference projector pattern. To this end, we projecting a grid of squares onto the planar facet π, and acquiring an image of the same.

The warped sinusoidal patterns used in the experiment are included in the image insets of Figure 12, and obtained using the aforementioned procedure.

The process of super resolution begins with the acquisition of images iθ(x,y) under the phase-shifted illumination patterns shown below:

| pθx,y=12+12sin2πη0π21x+π22y+π23π31x+π32y+π33+θ | (33) |
| --- | --- |
| wherein η0=16cycpixel , θ∈0,π2,π,3π2, x∈1,1400y∈1,1050 |  |

The camera images acquired under the aforementioned illumination patterns are processed using the demodulation strategy outlined in Section-3(A), to obtain a super resolved image of the scene. The result is disclosed in Panel-1 of  Figure 6. Inspection of the image insets confirms the ability to super resolve spatial detail in a macroscopic scene that exhibits topographic variations.

Figure 16 	Spatial Frequency Response (SFR) plots

The Spatial Frequency Response (SFR) plots of Figure 16 aid in the quantitative assessment of resolution gain. Comparison of the red/blue plots suggest a marked/marginal resolution gain in the modulation direction and its orthogonal orientation. The latter is an artifact of the dynamic range adjustment process that is used to match the visual appearance of the reconstructed image to the baseband image. A resolution gain of 1.6602 is observed in the modulation direction. The number is obtained as the ratio of the cutoff frequency before/after super-resolution. The cutoff frequency is defined as the spatial frequency for which the modulation strength falls to 0.02.

- Super-resolving optics with spatially varying blur

Figure 17 	Super-resolving singlet using a coincident stereo arrangement

Details of the coincident stereo apparatus used to demonstrate resolution improvement in a collection optic afflicted with space-variance, is included in Figure 17.

A 45°pellicle beam splitter and standard alignment techniques are used to ensure that the COP’s of the camera and projector are lined up. Details on the alignment procedure adopted in this work will be made available in the supplementary material.

Figure 18 	Measured PSF at select image field locations

The image insets of Figure 18 illustrates the severity of space-variance in the camera optical blur. The images represent the response of the collection optics to a grid-of-dots (size of each dot  270 µm × 270 µm) displayed on a 24" LCD monitor.

The process of super resolution begins with the acquisition of images iθ(x,y) under the phase-shifted illumination patterns shown below

| pθx,y=12+12sin 2πξ0x+θ pθx,y=12+12sin 2πη0y+θ | (34) |
| --- | --- |
| wherein ξ0,η0=16cycpixel , θ∈0,2π3,4π3, x∈1,1920y∈1,1080 |  |

Figure 19 	Results from attempt to super-resolve a singlet using sinusoidal patterns in the coincident stereo arrangement of Figure 16

The camera images acquired under the aforementioned illumination patterns are processed using the demodulation strategy outlined in Section-3(A), to obtain a super resolved image of the resolution target. The result is disclosed in Figure 19.

Inspection of the image insets confirms that space-variance in the optical blur is not an impediment to super-resolution. It also confirms that linearity (not space-invariance) is the principal requirement for super-resolution using heterodyning. The aforementioned view broadens the scope of super-resolution techniques to accommodate a much larger class of imagers not limited to well corrected optics.

A visual assessment of the number of resolvable bars in each USAF target, hints at a resolution gain of 4 elements. The knowledge that successive elements in the USAF target differ in resolution by 21/6 may be used to ascertain an empirical resolution gain of 1.5874.

Key observations

Field dependent resolution gain: A comparison of the modulation strength of element-6 in each colored inset confirms the field dependence in the resolving power of the engineered imager. The most visible improvement appears near the optical axis (blue inset), while the least visible improvement appears at the edge of the image field (red & green insets). The anisotropy in resolving power can be traced back to the field dependence in the resolving power of the baseband PSF.

Uneven brightness: This may be confirmed by comparing the brightness of the central portion of the super resolved image to its surroundings. The behavior may be traced back to the field dependence of the DC (zero-frequency) response of the camera optical blur. It is observed that the baseband image of a constant albedo target exhibits intensity variations in accord with the severity of space-variance in the camera optical blur.

Ghost artifacts: The impact of ghosting on the quality of reconstruction is most apparent in the red insets. The severity of ghosting increases with increasing distance from the optical axis. In addition, the direction of ghosting appears to depend on the modulation direction. Both observations are consistent with the findings reported in Section-3(A)2.

In summary, space-variance in the optical blur introduces

- field dependence in the resolving power of the computationally engineered imager, and

- ghost artifacts in the super-resolved image.

In Section-5, we examine alternative illumination/reconstruction strategies for simultaneously overcoming the above issues.

D. Limitations

A constant criticism levied against heterodyning based super-resolution schemes is the purported need for the cutoff frequency of the illumination optics to exceed the cutoff frequency of the imaging optics. This begs the obvious question: why not use the illumination optics for imaging?

We observe that the criticism is ill-conceived as the optical design tradeoff associated with the illumination optics are markedly different from those associated with the imaging optics. The illumination optics need only project a specific set of frequencies, a task that can be accomplished using a variety of techniques including sparse-aperture synthesis. In contrast, the imaging optics is by design required to resolve extended objects with spatial frequencies spanning the entire optical passband.

- Depth estimation using sinusoidal patterns

The proposed approach to recovering depth is a straightforward adaptation of Phase Measurement Profilometry (PMP) techniques [35] to macroscopic scales. The principal difference lies in the additional restrictions (bZ=0) that are imposed on the placement of the camera and projector. The restriction helps expand the scope of PMP techniques to accommodate a broader class of illumination patterns such as warped-sinusoids.

PMP approaches recover densely sampled topographic information from images of a scene illuminated by one or more periodic sinusoidal patterns. Depth estimation is predicated on the observation of scene dependent phase deformations in the camera image of the projected illumination pattern. It was established in Section-2(C) that a select combination of stereo arrangements and sinusoidal patterns support phase modulation of a carrier (Eqs.(21) & (22)), as favored by PMP.

Examination of Eqs.(21) & (22) confirms that the inverse of the scene depth (W-1) is embedded in the instantaneous phase φ(u,v) of the detected illumination pattern. This observation constitutes the basis of a depth estimation strategy.  Topographic information is recovered from the phase-map φ by eliminating the linear phase term and the constant phase offset φ0.

The process begins with the acquisition of images under phase shifted sinusoidal/warped-sinusoidal patterns pθx,y : θ∈0,π2,π,3π2 of frequency ξ0,η0. The camera images are then digitally recombined to obtain the cosine/sine modulated images of the scene, as outlined in Eqs.(26)– (28). The resulting expressions are disclosed below:

| icosx,y≝Bx,ycosφx,yrx,y | (35) |
| --- | --- |
| isinx,y≈¦Bx,y¦sinφx,yrx,y | (36) |
| wherein φx,y≈2πκoξ0x+η0y+φ0+2πZ-1κdξ0bX+η0bY |  |

Closer inspection of the expression for the cosine/sine images in Eqs.(35)-(36) exposes omissions: blurring due to the imaging optics, and the phase distortion due to the illumination blur.

As is common practice in PMP, we disregard the effect of camera optical blur. The practice is admissible when the primary objective is to recover qualitative instead of quantitative depth information, as is the case with the present work. The assumption of well-corrected illumination optics allows us to omit phase distortion due to the illumination blur.

The trigonometric identity tan-1sinQ-PcosQ-P=mod(Q-P,2π) may be used to isolate the scene-dependent component of the phase from the full phase map φ. Substituting P=2πκoξ0x+η0y+φ0 and Q=φx,y=P+2πκdZ-1ξ0bX+η0bY into the trigonometric identity, yields the following expression for the scene-dependent phase:

| φwrappedx,y≝mod2πκdZ-1ξ0bX+η0bY,2π | (37) |
| --- | --- |

The 2π phase ambiguity in Eq.(37) emerges from the phase ambiguity associated with the arctangent function. In order to resolve the phase ambiguity, we employ two sets of illumination patterns whose spatial frequencies are harmonically related.  The first set of sinusoidal patterns with spatial frequency (ξlow,ηlow) are designed to yield a phase-map φlow(x,y) that is free of wrapping artifacts. The second set of sinusoidal patterns with spatial frequency (ξhigh=Fξlow,ηhigh=Fξlow) yield a second phase-map φhigh(x,y) that is unwrapped using  φlowx,y, as shown below:

| φunwrapped= φhigh+2π roundF φlow-φhigh2π | (38) |
| --- | --- |

Figure 20 	Estimating depth in canonical/collocated stereo arrangement

The above unwrapping scheme relies on the fact that the scene dependent phase deformations induced by parallax vary linearly with the spatial frequency of the illuminating pattern. A topographic map of the scene can be obtained by computing the per-pixel inverse of the unwrapped phase-map, i.e., Z∝φunwrappedx,y-1. Figure 20 summarizes the key steps in the proposed workflow for estimating scene depth in canonical/collocated stereo arrangements.

- Advantages over PMP

As stated previously, the additional restrictions on camera, projector placement imposed by the proposed approach allows the use of a broader class of patterns for depth estimation. An example is the use of warped-sinusoidal patterns in collocated stereo arrangements. The warped structure of the sinusoidal illumination pattern compensates for the relative rotation between the camera and projector optical axes. In practice, it is observed that the warping helps avoid fringe bunching/aliasing in the detected illumination pattern. It also helps circumvent the need for an extensive intrinsic/extrinsic calibration of the stereo arrangement, as in standard PMP. Additional details are available in Section 6.3 of [4].

- Experimental validation

In the interest of brevity, we examine a single experimental result demonstrating the feasibility of depth estimation using warped-sinusoidal patterns. A more exhaustive list of experiments validating the proposed approach in a variety of monostatic and bistatic arrangements is compiled in [4, 44] and [47].

We re-examine the collocated stereo apparatus of Figure 15 for the purpose of this discussion. The process of scene recovery begins with the acquisition of images iθ(x,y) under the phase-shifted illumination patterns shown below:

| pθlowx,y=12+12sin2πξ0lowπ11x+π12y+π13π31x+π32y+π33+θ pθhighx,y=12+12sin2πξ0highπ11x+π12y+π13π31x+π32y+π33+θ | (39) |
| --- | --- |
| wherein ξ0low=12592cycpixelξ0high=118cycpixel , θ∈0,π2,π, 3π2, x∈1,1400y∈1,1050 |  |

The camera images acquired under the aforementioned set of illumination patterns are processed using the workflow of.

The frequency of the demodulation matches the frequency of the illumination pattern (ξ0,η0), since the warping of the sinusoidal illumination pattern accounts for differences in the magnification of the imaging and illumination paths, i.e.,  κo=1 in Eq.(36).

The first set of sinusoidal patterns with spatial frequency (ξ0low ,0) yield a phase-map  φlowx,y that is devoid of phase wrapping artifacts. The second set of sinusoidal patterns with spatial frequency (ξ0high ,0) yields a second phase-map φhigh(x,y), which is unwrapped using  φlowx,y. The topographic map of the scene is derived from the unwrapped phase is illustrated in Panel-1 of Figure 6.

The range resolution of a topographic measurement technique is an important figure-of-merit that quantifies the smallest discernible difference in scene depth (ΔZ). The image insets in Panel-1 of Figure 6 provide a visual assessment of the range resolution of the collocated stereo apparatus. The red stripe overlaid on the camera image depicts one of the many isophase contours of the sinusoidal illumination pattern. Inspection of the blue inset confirms the presence of phase distortion in a single stripe pattern, as it strikes a 1mm thick LEGO fence at a standoff distance of 1m from the camera COP.

- Why not sinusoids?

The overwhelming popularity of sinusoidal illumination patterns in super-resolution using heterodyning stems from the inherent simplicity of the reconstruction algorithm and the intuition associated with the modulation and demodulation process. In practice, the presence of artifacts in the reconstructed image limits the utility of sinusoids to well-corrected optics characterized by largely space-invariant blur. As stated previously, the use of countably finite sinusoidal patterns in conjunction with space-variance in the optical blur introduces

- field dependence in the resolving power of the computationally engineered imager, and

- ghost artifacts in the super-resolved image.

The next section examines alternative illumination/reconstruction strategies for overcoming the above issues.

- ALTERNATIVES TO SINUSOIDAL ILLUMINATION

Heterodyning is an inescapable consequence of spatially patterning the illumination that is incident on the object volume. The fact may be confirmed by re-examining the light transport model derived in Section-2(B). Inspection of the expression for the detector irradiance in Eq.(19) indicates that the camera image is a blurred representation of the geometric image of the scene that is subject to amplitude modulation.

On account of the linearity of the light transport mechanism, the expression for detector irradiance disclosed in Eq.(19) may be readily extended to accommodate any combination of sinusoids, and in general arbitrary patterns. This suggests that any spatial pattern besides flood illumination will likely heterodyne object information into the passband of the collection optics. The observation serves as the guiding principle for our subsequent efforts to design illumination patterns. We specifically borrow ideas from coding theory and communication theory in designing illumination patterns with desirable properties. The carefully chosen patterns help restore the heterodyned frequencies that are otherwise smeared across the imager passband to their correct position outside the passband. The smearing results from the modulation of object detail with the multitude of frequencies in the illumination pattern.

- Assumptions

The remainder of this document limits its attention to super-resolving macroscopic scenes in multiple orientations. This imposes restrictions on the placement of the camera and projector. Previous discussions inform us of two stereo arrangements that support super-resolution in multiple orientations. These include a coincident stereo arrangement, and a bistatic stereo arrangement observing 2D scenes lacking depth variation from the camera and projector perspective.

Figure 21 	Super-resolution by spread spectrum modulation

Without loss of generality, subsequent discussions assume that the illumination pattern incident on target is of finite extent (MΔ×NΔ) and expressible as a superposition of non-overlapping spots of size Δμm×Δμm. Such a pattern admits the following decomposition

| px,y=k,l=0M-1N-1ψk,l gx-kΔ,y-lΔ | (40) |
| --- | --- |

wherein the term g represents a top-hat function (rect function) whose intensity is modulated by the discrete sequence ψ.

The aforementioned constraints on the illumination pattern may be met using multiple approaches such as the use of well-corrected optics to project a spatial pattern, using diffractive optical elements as pattern generators, and rapidly scanning a modulated Gaussian beam across the imager field of regard. The last two options are attractive in that they allow us to dispense with need for wide-field projection optics with exacting spot size requirements.

- Mathematical Preliminaries

Our analysis begins by re-examining the expressions disclosed in Eqs.(23)&(24), for the detector radiance in a coincident and bistatic stereo arrangement. These expressions may be generalized to accommodate an arbitrary superposition of sinusoids on account of the linearity of the light transport mechanism. The result is included below:

| ix,y=pu,v ru,v hcamx-u,y-v;u,v dudv | (41) |
| --- | --- |

The term pu,v represents the camera image of the projected pattern, while ru,v represents the geometric image of the scene under uniform illumination. Please note in this case the term p(u,v) subsumes the effect of blurring due to the illumination optics. This blur is space-invariant on account of the constraints imposed in previous paragraphs. The space-variance arising from defocus may be disregarded when dealing with 2D scenes. Additionally, the use of beam scanning solutions for illuminating the object volume allow us to disregard space-variance in the illumination blur, on account of the limited beam divergence at standoff.

In the event that the image sensor out-resolves the imaging optics, the camera image of Eq.(41) may be resampled so that its inter-sample-spacing matches that of the illumination pattern (Δ), and the respective sampling grids are aligned. The expression for the re-sampled camera image may be re-cast in the projector coordinates by using the coordinate mapping of Eq.(9), using the infinite homography  H∞=KK-1. The choice reflects restrictions imposed by the coincident and bistatic stereo arrangements considered in this discussion.

The final expression for the resampled camera image is provided below:

| ix,y=κo-2pu,v ru,v hcamx-u,y-v;u,v dudv | (42) |
| --- | --- |

The terms r, hcam represent the geometric image of the scene and the camera optical blur, as expressed in the projector coordinates. The operations encapsulated in Eqs.(41)-(42) are realized in practice by warping the camera image using the infinite homography H∞=KK-1. The warp accommodates differences in the transverse magnification of the imaging and illumination paths, and differences in the sampling phase of the imaging and illumination grids.

The above expression is the starting point for the investigation into alternative illumination patterns and reconstruction strategies.

A. Super-resolution by spatio-temporal coding

The first approach draws inspiration from the spread spectrum modulation schemes in communication theory. The concept is illustrated in Figure 21. Before delving into the mechanics of its operation, we provide an intuitive description of the approach.

The idea is to assign a temporally unique code to neighboring scene points by patterning the illumination. Following optical blurring, these codes will smear into each another. The smearing is undone by demodulating each pixel with its corresponding code.

The success of the approach hinges on the uniqueness of the code. Codes such as CDMA spreading codes and perfect binary sequences exhibiting compact correlation properties (delta correlated to be precise) are suitable candidates. The smearing of codes resulting from optical blurring can be undone by exploiting the fact that the codes are un-correlated with translated copies of itself. This property forms the basis of the reconstruction strategy described below.

As stated previously, super resolution using heterodyning is predicated on the observation of unresolved spatial frequencies in the image ix,y obtained under patterned illumination. Fourier analysis of the product pu,v ru,v in Eq.(42) confirms that modulation smears the object spectrum across the optical passband. The challenge lies in undoing the smearing induced by amplitude modulation. The proposed solution exploits the modulation diversity afforded by circularly shifting the illumination pattern pu,v, in increments of the feature size Δ. The super resolved image is assembled by demodulating the temporal sequence of images acquired under translates of the illumination pattern pu,v, and accumulating the result.

The reconstruction process outlined above mimics the operation of a correlation receiver. The expression for the reconstructed image is disclosed below:

| ireconx,y=κo-2s,t=0M-1N-1ps,tx,yps,tu,vru,v hcamx-u,y-v;u,v dudv | (43) |
| --- | --- |
| wherein |  |
| ps,tx,y=k,l=0M-1N-1ψmodk-s,Mmod(l-t,N), gx-kΔ,y-lΔ |  |

The summation in Eq.(43) may be simplified by incorporating the definition of ps,tx,y from Eq.(40). The result is disclosed below

| s,t=0M-1N-1ps,tx,yps,tu,v=k,l=0M-1N-1m,n=0M-1N-1s,t=0M-1N-1ψmodk-s,Mmod(l-t,N),ψmodm-s,Mmod(n-t,N), gx-mΔ,y-nΔ gu-kΔ,v-lΔ | (44) |
| --- | --- |

The term within the square bracket represents the circular- correlation of the discrete sequence ψ. In the special case that the discrete sequence is delta-correlated, Eq.(44)simplifies as follows:

| s,t=0M-1N-1px-sΔ,y-tΔpu-sΔ,v-tΔ=k,l=0M-1N-1 gx-kΔ,y-lΔ gu-kΔ,v-lΔ | (45) |
| --- | --- |

Further simplification is possible by exploiting the finite extent of the top-hat function g. It is observed that the terms gx-kΔ,y-lΔ, gu-kΔ,v-lΔ within the summand are strictly non-zero over the intervals  x∈kΔ±12Δ, y∈lΔ±12Δ and  u∈kΔ±12Δ, v∈qΔ±12Δ. As a result, the product of these functions is non-zero over x-u≤Δ , y-v≤Δ. The above finding may be recast in terms of the definition of the top-hat function g, to obtain the following result,

| s,t=0M-1N-1px-sΔ,y-tΔpu-sΔ,v-tΔ=gx-u2,y-v2 | (46) |
| --- | --- |

Incorporating the above result into the expression for the reconstructed image yields the following revised expression for the reconstructed image

| ireconx,y=ru,vgx-u2,y-v2hcamx-u,y-v;u,v dudv | (47) |
| --- | --- |

Notice that the reconstructed image bears a strong resemblance to the image acquired with the engineered PSF

| hengdx,y;u,v=gx2,y2hcamx,y;u,v≈gx2,y2 | (48) |
| --- | --- |

The approximation follows from the fact that the transverse extent of the top-hat function g is almost always smaller than the transverse extent of the optical PSF hcam. This suggests that the computationally engineered imager has near isotropic resolving power throughout the image field. In other words, the resolving power of our computational imager is decoupled from the resolving power of the collection optics.

According to Eq.(48), the resolving power of the computationally engineered imager is twice the support of the top-hat function g, i.e., 2Δ. This claim is consistent with the fact that the finest frequency in the illumination pattern is limited to 0.5Δ-1.

- Limitations

In practice, the number of scene points in the imager field of regard could far exceed the number of unique codes available. In such cases, we recommend reusing codes in accord with the worst case spot size of the camera optical blur. A simple method for code reuse involves tiling the code to fill up the illumination field. The tiling introduces a periodicity in

the illumination pattern, so that the illumination pattern admits a Fourier series decomposition. Illuminating the scene with the said pattern amounts to simultaneous modulation with a set of harmonically related sinusoids in multiple orientations. The challenge lies in disambiguating the contributions of each of the carrier components. The use of a stochastic code with compactly supported correlation helps isolate the contribution of each carrier component.

- Experimental validation

The feasibility of super-resolution using spatio-temporal coding is verified by attempting to improve the resolving power of a well-corrected optic observing an ISO chart. The camera and projector used in the experiment are identical to those used in Figure 15. The difference lies in the use of bistatic stereo arrangement for the present experiment.

The standoff distance from the camera to the target is ≈0.67m, while that from the projector is ≈. The short throw of the projector necessitated the movement of the projector closer to the target to realize higher resolution gains.

The ISO target is super-resolved by exploiting modulation diversity afforded by the use of a single pseudorandom binary pattern and a continuum of phase shifts. The pseudorandom is obtained by tiling a 15×17 minimum length sequence to fill up the illumination field.  Care was taken to ensure that the size of the binary pattern exceeded the spot size of the camera optical blur.

A super-resolved image is assembled by carefully demodulating the temporal sequence of images captured under 17×15=255 shifts of the master illumination pattern. The exposure time associated with each measurement is 62.5 msec. An animation of the reconstruction process is included in the supplementary material [45].

Results from the experiment are included in Figure 23. Closer inspection of the chart elements indicates a resolution gain in excess of 3.5. A more accurate estimate of 3.8× is obtained from the SFR plots of Figure 22. Examination of the SFR plots reveals variations in the saggital and meridional SFR of the engineered imager. It is observed that the anisotropy stems from the anisotropy in the SFR of the projection optics.

Figure 22 	Spatial Frequency Response (SFR) plots

Figure 23 	Results from attempt to super-resolution a well-corrected optic using a stochastic illumination pattern.

(please zoom-in for a closer view)

Pleae

The images in Panel-2 of Figure 6 are also obtained as part of this experiment.

- Broader impact

A key ingredient in super-resolution using stochastic illumination is the correlation receiver used in assembling the super-resolved image. The notion exploits the modulation diversity afforded by the use of a single spatial pattern and a continuum of phase shifts. This notion can be readily extended to support any spatial pattern suggesting the possibility of super-resolution using any spatial pattern. The corresponding expression for the engineered PSF is given by
hengdx,y;u,v=χpp(x-u,y-v)hcamx,y;u,v, where  χpp denotes the auto-correlation of the master illumination pattern. In the special case that the master illumination pattern is a pure sinusoid, the auto-correlation is also a sinusoid with the same periodicity. In this sense, the correlation receiver may be viewed as a general reconstruction strategy supporting a wide class of patterns ranging from sinusoids to stochastic patterns.

It is observed that the resolving power of the aforementioned computational imager is limited by the support of the auto-correlation of the master illumination pattern. The claim is consistent with the observation that flood illumination yields no improvement in resolution, and delta-correlation yields maximal improvement in resolution.

- Super-resolution using lattice illumination

Migrating from sinusoidal illumination patterns to stochastic illumination patterns has allowed us to engineer computational imagers whose resolving power is decoupled from that of the collection optics. Still, undersampling at the sensor presents a challenge, even more so in the case of spread spectrum modulation. The final approach examined in this work tackles the aforementioned problem.

It is common knowledge that the resolution requirements for imaging a sparse collection of point emitters is substantially weaker than the requirements for imaging extended objects. In the former case, the point emitters need only be separated by the two-point resolution for exact reconstruction. In the latter case the imaging optics needs to resolve all spatial frequencies that make up the extended object.

The aforementioned distinction between imaging sparse/extended objects forms the basis of our most expansive super-resolution strategy.

The idea is convert the extended object volume into a sparse collection of secondary point emitters by using patterned illumination. The pattern of choice is a single spot or a collection thereof, which is referred to as lattice illumination in this work. In the case of lattice illumination, care must be taken to ensure that the lattice spacing avoids overlap between the image of adjacent spots.

Figure 24 	Super-resolution by spot(s) scanning/lattice illumination

Following optical blurring, light from each illuminated object spot is distributed over one or more sensor pixels. By reintegrating the distributed light, we can simultaneously decouple the imager resolving power from the constraints imposed by the collection optics and the detector.  Figure 24 illustrates the concept for a single spot.

The resolving power of the aforementioned computational imager is strictly limited by the spot size of the illumination optics. The expression for the reconstructed image, in the case of spot scanning, is disclosed in Eq.(49).

| irecons,t∝s,t=0M-1N-1ps,tu,v ru,vhcamx-u,y-v;u,vdxdydudv | (49) |
| --- | --- |
| wherein ps,tu,v≝gsΔ,tΔu∈sΔ±12Δ,v∈tΔ±12Δ 0otherwise |  |

Figure 25 	Super-resolving a singlet using lattice illumination

The inner integral represents light aggregation over multiple detector pixels, subsequent to imaging. In practice, the limits of integration need only be limited to an area larger than the worst case spot size of the collection optics. The operation mimics light integration at a single pixel detector with an active area that exceeds the worst case spot size.

The inner integral in Eq.(49) represents the integrated intensity of the camera optical PSF, which by definition is a field-independent constant. Incorporating this fact into the expression for the reconstructed image yields the following:

| irecon[s,t]∝gu-sΔ,v-tΔru,vdudv | (50) |
| --- | --- |

Notice that the reconstructed image bears a strong resemblance to the image acquired with the engineered PSF g corresponding to the light distribution in a single spot.

The resolving power of the aforementioned computational imager is strictly limited by the feature size of the illumination pattern, which in this case is Δμm.

- Connection with super-resolution using sinusoidal patterns

Both the single spot and lattice illumination patterns admit a Fourier decomposition, so that illumination of the scene amounts to simultaneous modulation with multiple sinusoids in different orientations. The challenge lies in disambiguating the contributions of each of the sinusoidal components. The modulation diversity afforded by scanning the spot / shifting the lattice helps isolate the contribution of each sinusoidal component.

Figure 26 	Space-variance in the camera PSF over illuminated field

Connection with super-resolution using spatio-temporal coding

The temporal sequence of lattice illumination patterns used to illuminate the scene may be interpreted as a unique binary temporal code that is assigned to each scene point in the illuminated field. The connection suggests that a correlation receiver may also be used to assemble the reconstructed image, in lieu of the reconstruction strategy outlined above.

- Connection with super-resolution microscopy

The proposed technique could be viewed as an adaptation of super-resolution techniques [52-56] such as PALM/STORM, with the broader goal of accommodating space-variance in the optical blur and detector undersampling.

- Experimental validation

The apparatus shown in Figure 25 is tasked with the objective of producing high quality imagery using a 25 mm double convex lens, and periodic pulse train illumination. The single lens imager is comprised of multiple optical components, as disclosed in the figure. The extension tube and spacers aid in controlling the position of the plane of sharp focus. The diameter of the iris mounted in front of the lens controls the severity of aberrations in the camera image.

The use of a laser beam scanning projector helps dispense with the need for high quality projection optics. The bistatic arrangement of the camera and the projector accommodates differences in their transverse magnification. Not shown in the figure is a moving diffuser (3M P/N: NVAG829233) that helps minimizes the occurrence of speckle artifacts in the camera image.

The image of Figure 26 illustrates the space-variance in the PSF of the single lens imager, over the illumination field. The image spans 499 × 925 pixels (1.1 mm × 2.03 mm), and represents the response of the single lens imager to a grid-of-squares (size of each square = 3 × 3 projector pixels) projected by the LBS. It can be observed that the blur spots are asymmetrically distributed about the center of the image field. The asymmetry is attributed to the offset projection of the LBS.

The process of super resolution begins with the acquisition of images under integer pixel translates of the lattice illumination pattern disclosed below:

| ps,tx,y=k=1,l=1848,480ψk-s,l-t g(x-kΔ,y-lΔ) wherein ψk,l ≝δ[ modk,21,modl,21 ] | (51) |
| --- | --- |

The discrete sequence ψ represents a lattice of spots with a periodicity of 21 pixels in the horizontal and vertical directions. The lattice spacing of 21 pixels was empirically determined to be sufficient to avoid overlap in the image of adjacent light spots. The 441 (=21×21) camera images acquired under lattice illumination, are resampled using a geometric warp that accommodates difference in magnification of the imaging and illumination paths, and compensates for field distortion in the projected pattern. The resampled camera images are multiplied with the respective illumination patterns, and the product accumulated to obtain the super resolved image. Mention that the reconstruction algorithm is a correlation receiver and not.

The outcome of super resolution is documented in Panel-3 of Figure 6. Inspection of the insets confirms that the proposed super resolution strategy proposed here may be used to produce high-quality imagery free of the annoying ghost artifacts that afflicted previous attempts (Section-3(C)2) to super resolve a single lens imager.

Figure 27 	Spatial Frequency Response (SFR) plots

The SFR plots of Figure 27 help the claim of isotropic resolving power in the super-resolved image field.  It is observed the saggital and tangential resolution of the engineered imager are largely field-independent. The asymmetry in the saggital and tangential resolving powers is attributed to the elliptical structure of the laser light spot.

- Multiscale reconstruction

Unlike super-resolution using sinusoidal/stochastic patterns each image acquired under lattice illumination may be independently reconstructed to recover scene information at select field locations. This implies that a single image acquired under lattice illumination may be reconstructed to directly provide a low-resolution (decimated to be more precise) reconstruction of the scene. The pixel pitch of the reconstruction is determined by the periodicity of the lattice, whereas the fill factor is determined by the size of the illuminated spot.

Additional images merely provide the sampling diversity needed to assemble partial reconstructions of increasing resolution. The process forms the basis of a multiscale reconstruction strategy, and is illustrated in Panel-4 of Figure 6.

The partial reconstructions in the image pyramid are obtained by progressively quadrupling the number of observations used to assemble the reconstructed image. The quadrupling represents an octave improvement in the sampling density.

Alternatively, one can digitally interpolate the intermediate images to obtain a synthetic reconstruction of the scene, albeit at full-resolution. However, the periodic structure of the lattice introduces sampling artifacts in the reconstruction. The use of a pseudo-random lattice pattern alleviates the problem. In practice, it is observed that as many as 50% of the observations are sufficient to obtain a high fidelity reconstruction of the scene. An example is provided in the insets of Panel-4. The peak signal to noise (PSNR) value of the mean squared error between the exact reconstruction and the interpolated partial reconstruction is also indicated for comparison.

Prior information about the target (such as sparsity in a known basis) could be used to lower the number of observations needed to assemble the high fidelity reconstruction.

- Broader impact

A unique characteristic of super-resolution using lattice illumination is that it is agnostic to the structure of the PSF of the collection optics. This provides us with an opportunity to encode novel information into the camera side PSF. As an example one could attempt to encode range information using the double helix PSF [57,58].

The monostatic variant of the stereo arrangement used in Figure 25 bears a striking resemblance to scanning units in a variety of active EO (electro-optic) sensors such as LADAR, and gated-viewing imagers. The principal distinction between these systems is the use of patterned illumination. It is envisioned that the capabilities of many EO sensors can be augmented to acquire high resolution imagery, with suitable modifications.

- SUMMARY

The notion of active computational imaging espoused in this work is centered on the premise that imaging under patterns illumination affords capabilities not achieved using classical approaches. The present work has examined this notion within the context of circumventing resolution limits at macroscopic scales. The mathematical tools required to comprehend and extend the ideas described in this work have been furnished as part of the investigation. This includes a comprehensive model for light transport that assimilates established principles from physical optics and computer vision into a broader framework for imaging under patterned illumination.

A taxonomy of select active computational imaging concepts examined in this work, are summarized in Figure 28. The numerical code assigned to each branch in the map represents a specific combination of restrictions imposed on the illumination pattern, collection optics and image sensor.

Please note that the notion of engineering a prescribed PSF using sinusoidal illumination is not examined in this manuscript. Interested readers are referred to [4,64] for more details.

Key takeaways from the broader investigation are summarized below:

The camera image of an illumination pattern in any active stereo arrangement will exhibit amplitude and/or phase modulation. The amplitude modulation in isolation is essential to super-resolution and contributes to improved transverse resolution). The phase modulation in isolation is essential to depth estimation and contributes to improved longitudinal resolution.

A variety of stereo arrangements support super-resolution and/or depth estimation. One embodiment includes collocated stereo arrangements wherein the projector COP is embedded within the camera entrance pupil plane. Illumination patterns whose iso-intensity lines are aligned with the epipolar lines on the camera side do not exhibit scene dependent phase deformations, and are consequently best suited for super resolution. On the other hand, illumination patterns whose iso-intensity lines are not-aligned/perpendicular to the epipolar lines on the camera side exhibit scene dependent phase deformations, and are consequently best suited for depth estimation.

A second embodiment includes coincident stereo arrangements wherein the camera and projector optical axes and COP are aligned. These stereo arrangements support super-resolution in any orientation.

The collocated/co-incident stereo arrangements discussed in this work bear a striking resemblance to the scanning units in electro-optics sensors such as LADAR, gated-viewing cameras, structured light scanners and profilometers. It is envisioned that the capabilities of these sensors may be augmented to acquire higher resolution imagery, with suitable modifications.

Figure 28 	Taxonomy of active computational imaging concepts explored in this work

Key takeaways from the investigation into super-resolution are summarized below:

Space-variance in the imager blur is not an impediment to super-resolution. The improvement in transverse resolution is effected by a computational engineering of the imager PSF.  Resolution gain manifests as a reduction in the spot size or the central lobe width of the engineered PSF. In the case of sinusoidal patterns, maximal/zero resolution improvement occurs in/orthogonal to direction of modulation.

Any spatial pattern besides flood illumination will heterodyne unresolved spatial frequencies into the passband of the imaging optics. Exact restoration of the heterodyned frequencies is made possible by exploiting the modulation diversity afforded by the use of a single pattern and a continuum of phase shifts. The aforementioned reconstruction process mimics the operations of a correlation receiver.

A stochastic illumination pattern used in conjunction with a correlation receiver allows us to engineer computational imagers whose resolving power is decoupled from that of the collection optics. The resulting computational engineer exhibits isotropic resolving power throughout the image field.

Undersampling at the sensor complicates attempts to restore the heterodyned frequencies when using sinusoidal patterns paired with an AM receiver OR stochastic patterns paired with a correlation receiver.

Neither sinusoidal nor stochastic patterns support reconstruction from partial observations as the reconstruction problem is ill-posed. The ill-posed nature of the reconstruction problem stems from the ambiguity in restoring the heterodyned frequencies to their correct position outside the optical passband.

Super-resolution techniques relying on spot/lattice illumination can be used to engineer imagers whose resolving power is fully decoupled from the constraints imposed by the collection optics and sensor. The resulting computational engineer exhibits isotropic resolving power throughout the image field. Unlike other active super-resolution techniques, a single observation of the scene under lattice illumination yields a low-resolution representation of the scene. Lastly, super-resolution techniques relying on spot/lattice illumination support multiscale reconstruction and reconstruction from partial information.

- Research avenues

The chief shortcomings of our active computational imaging approach include: (a) sacrificing temporal bandwidth in an attempt to squeeze more information into the limited optical/detection bandwidth of the imager, and (b) the resolution on target is fundamentally limited by the feature size of the illumination pattern, as seen on target.

The loss in bandwidth may be minimized by borrowing ideas from compressed sensing, specifically the notion of object sparsity in a basis set that could be identified using the low-resolution image of the scene. It is anticipated that sparse-signal reconstruction techniques could be used in conjunction with lattice illumination to recover a super-resolved image of the scene, using far fewer observations than necessary for exact reconstruction.

Current efforts are devoted towards tackling the second problem of feature size on target. Two distinct approaches to minimizing the feature size on target are being examined. The first of these approaches exploits recent innovations in engineering optical beams that can squeeze light into a region smaller than the diffraction limit [59]. The reduction in spot size is accompanied by an increase in side lobes, which with some effort could be moved away from the central lobe. The reduced spot size is useful for computational resolution enhancement, while the separation of the side lobes avoids coupling light into the image of the central spot. Both capabilities come at the expense of SNR, since an appreciable amount of light energy is redirected towards the side lobes.

The second approach to controlling the feature size on target involves the use of a lensless projection scheme to shape the field distribution in the object volume. The approach contrasts the current practice of shaping the irradiance component. It exploits the notion that free space propagation preserves the bandwidth of a propagating field that is setup using a Spatial Light Modulator (SLM). It can be shown that the feature size on target is limited only by the pixel pitch of the SLM.

A few other avenues for research are suggested below. There is a strong connection between the multiplicative structure of the engineered PSF in our work, and the PSF’s encountered in interferometric two-pupil synthesis [60] and sparse aperture synthesis [61]. Likewise, a non-physical/computational pupil function could be associated with our engineered PSF, suggesting a connection with pupil replication techniques [62-63].

Another avenue for research includes the examination of a holistic approach to the design of electro-optics systems featuring patterned illumination.

Funding Information.  U.S Army Research Laboratory (ARL) (W911NF-06-2-0035)

Acknowledgment. The authors wish to thank Dr. Vikrant Bhakta, Dr. Panos Papamichalis and Dr. Manjunath Somayaji for their valuable help in shaping some of the early ideas described in the work.

The work described in this paper was sponsored by the Army Research Laboratory and was accomplished under Cooperative Agreement Number W911NF-06-2-0035.  The views and conclusions contained in this document are those of the authors and should not be interpreted as representing the official policies, either expressed or implied, of the Army Research Laboratory or the U.S. Government.  The U.S. Government is authorized to reproduce and distribute reprints for Government purposes notwithstanding any copyright notation heron.

References

- D. Donoho, “Compressed sensing,” IEEE Trans. Inf. Theory 52(4), 1289–1306 (2006).

- E. J. Candès, J. Romberg, and T. Tao, “Robust uncertainty principles: exact signal reconstruction from highly incomplete frequency information,” IEEE Trans. Inf. Theory 52(2), 489–509 (2006).

- M. F. Duarte, M. A. Davenport, D. Takhar, J. N. Laska, T. Sun, K. F. Kelly, and R. G. Baraniuk, “Single-pixel imaging via compressive sampling,” IEEE Signal Proc. Mag. 25(2), 83–91 (2008).

- Rangarajan, Prasanna. "Pushing the limits of imaging using patterned illumination." PhD diss., SOUTHERN METHODIST UNIVERSITY, 2014.

- W. Lukosz and M. Marchand, “Optischen Abbildung Unter Uberschreitung der Beugungsbedingten Auflosungsgrenze,” Journal of Modern Optics, vol. 10, no. 3, pp. 241-255, Jul. 1963.

- M. G. Gustafsson, “Surpassing the lateral resolution limit by a factor of two using structured illumination microscopy,” Journal of microscopy, vol. 198, no. 2, pp. 82-7, May 2000.

- M. G. L. Gustafsson, “Nonlinear structured-illumination microscopy: wide-field fluorescence imaging with theoretically unlimited resolution,” Proceedings of the National Academy of Sciences of the United States of America, vol. 102, no. 37, pp. 13081-6, Sep. 2005.

- R. Heintzmann, T. M. Jovin, and C. Cremer, “Saturated patterned excitation microscopy--a concept for optical resolution improvement,” Journal of the Optical Society of America. A, Optics, image science, and vision, vol. 19, no. 8, pp. 1599-609, Aug. 2002.

- R. Heintzmann and G. Ficz, “Breaking the resolution limit in light microscopy,” Briefings in functional genomics & proteomics, vol. 5, no. 4, pp. 289-301, Dec. 2006.

- S. A. Shroff, “Structured Illumination Imaging,” PhD Thesis, University of Rochester, 2010.

- J. T. Frohn, H. F. Knapp, and A. Stemmer, “True optical resolution beyond the Rayleigh limit achieved by standing wave illumination,” Proceedings of the National Academy of Sciences of the United States of America, vol. 97, no. 13, pp. 7232-6, Jun. 2000.

- J. T. Frohn, “Super resolution Fluorescence Microscopy by Structured Light,” PhD Thesis, Swiss Federal Institute of Technology, 2000.

- J.-Y. Lin, R.-P. Huang, P.-S. Tsai, and C.-H. Lee, “Wide-field super resolution optical sectioning microscopy using a single spatial light modulator,” Journal of Optics A: Pure and Applied Optics, vol. 11, no. 1, p. 015301, Jan. 2009.

- A. Stemmer, M. Beck, and R. Fiolka, “Widefield fluorescence microscopy with extended resolution,” Histochemistry and cell biology, vol. 130, no. 5, pp. 807-17, Nov. 2008.

- A. Neumann, Y. Kuznetsova, and S. R. Brueck, “Structured illumination for the extension of imaging interferometric microscopy,” Optics express, vol. 16, no. 10, pp. 6785-93, May 2008.

- V. Micó, Z. Zalevsky, and J. García, “Optical Superresolution: Imaging Beyond Abbe’s Diffraction Limit,” Journal of Holography and Speckle, vol. 5, no. 2, pp. 110-123, Aug. 2009.

- M. G. L. Gustafsson, “Extended resolution fluorescence microscopy,” Current opinion in structural biology, vol. 9, no. 5, pp. 627–628, 1999.

- Z. Zalevsky and D. Mendlovic, Optical superresolution. Springer, 2004.

- E. Barrett, D. W. Tyler, P. M. Payton, K. Ip, and D. N. Christie, “New approaches to image super resolution beyond the diffraction limit,” Proceedings of SPIE, vol. 6712, p. 67120D-67120D-14, 2007.

- D. W. Tyler and E. B. Barrett, “Simulation of a passive grating-heterodyne super resolution concept,” Proceedings of SPIE, vol. 7094, pp. 709403-709403-11, 2008.

- A. Mudassar, A. R. Harvey, A. H. Greenaway, and J. Jones, “Band pass active aperture synthesis using spatial frequency heterodyning,” Journal of Physics: Conference Series, vol. 15, pp. 290-295, Jan. 2005.

- E. Sabo, Z. Zalevsky, D. Mendlovic, N. Konforti, and I. Kiryuschev, “Superresolution optical system with two fixed generalized Damman gratings.,” Applied optics, vol. 39, no. 29, pp. 5318-25, Oct. 2000.

- E. Ben-Eliezer, N. Konforti, and E. Marom, "Super resolution imaging with noise reduction and aberration elimination via random structured illumination and processing," Opt. Express  15, 3849-3863 (2007).

- E. Ben-Eliezer and E. Marom, "Aberration-free superresolution imaging via binary speckle pattern encoding and processing," J. Opt. Soc. Am. A  24, 1003-1010 (2007).

- Shirai, Y., "Recognition of polyhedron with a range finder." Pattern Recognition, 4, 243-250, (1972).

- Nevita, R. and T.O. Binford, "Structured descriptions of complex objects." Proceedings 3rd International Joint Conference on Artificial Intelligence, 641-647, (1973).

- R.I. Popplestone, C.M. Brown, A.P. Ambler and G.F. Crawford, "Forming models of plane-and-cylinder faceted bodies from light stripes," Proceedings 4th International Joint Conference onArtificial Intelligence, 664-668, (1975).

- A. Shpunt and Z. Zalevsky. “Three-dimensional sensing using speckle patterns,” March 2007. US Patent App. 12/282,517.

- Z. Zalevsky, A. Shpunt, A. Maizels, and J. Garcia. “Method and System for Object Reconstruction,” March 2006. US Patent App. 11/991,994.

- J. Salvi, J. Pages, and J. Batlle, “Pattern codification strategies in structured light systems,” Patt. Recogn. 37(4), 827–849 (2004).

- M. D. Altschuler, B. R. Altschuler, and J. Taboada, "Laser electro-optic system for rapid three-dimensional (3-D) topo- graphic mapping of surfaces," Opt. Eng. 20, 953-961 (1981).

- E. Horn and N. Kiryati, "Toward optimal structured light patterns," Image Vision Comput. 17, (2), 87‒97 (1999).

- M. Halioua and R. S. Krishnamurthy. Projection moire with moving gratings for automated 3-D topography. Applied Optics, 1983.

- M. Takeda and K. Mutoh. Fourier transform profilometry for the automatic measurement of 3-D object shapes. Applied Optics, 22(24):3977, Dec. 1983.

- V. Srinivasan, H. C. Liu, and M. Halioua. Automated phasemeasuring profilometry: a phase mapping approach. Applied Optics, 24(2):185, Jan. 1985.

- M. Chang and C. S. Ho. Phase-measuring profilometry using sinusoidal grating. Experimental Mechanics, 33(2):117–122, June 1993.

- P. S. Huang, S. Zhang, and F. P. Chiang, “Trapezoidal phase-shifting method for 3-d shape measurement,” SPIE Bellingham WA, vol. 5606, 2004.

- P. Jia, J. Kofman, and C. English, “Two-step triangular-pattern phase-shifting method for three dimensional object-shape measurement,” Optical Engineering, vol. 46, no. 8, 2007.

- Alexander Hornberg. 2006. Handbook of Machine Vision. Wiley-VCH.

- M. Aggarwal and N. Ahuja, “A Pupil-Centric Model of Image Formation”, Int. J. Comput. Vision 48, 3 (July 2002)

- R. Hartley, A. Zisserman, Multiple View Geometry in Computer Vision (Second Edition), Cambridge University Press, 2003.

- Y. Ma, S. Soatto, and J. Koeck An invitation to 3-D vision: From images to geometric models (Springer Verlag, 2004).

- A. W. Lohmann and D. P. Paris, “Space-Variant Image Formation,” Journal of the Optical Society of America, vol. 55, no. 8, pp. 1007-1013, Aug. 1965.

- P. Rangarajan, I. Sinharoy, P. Papamichalis, and M. Christensen, “Pushing the limits of digital imaging using Structured Illumination,” Proc. of the International Conference on Computer Vision, 1315-1322 (2011)

- Link to supplementary material: http://lyle.smu.edu/~prangara/pubs/AO_Manuscript/

- D.A. Vaquero, R. Raskar, R.S Feris, and M. Turk, "A projector-camera setup for geometry-invariant frequency demultiplexing", Proc. of Computer Vision and Pattern Recognition, 2082-2089 (2009)

- P. Rangarajan, V. Bhakta, M. Christensen, and P. Papamichalis, “Perspective imaging under structured light,” Proc. of the European Conference on Computer Vision, 405-419, Sep.2010

- L. Zhang and S.K. Nayar, "Projection defocus analysis for scene capture and image display," ACM Trans. Graph. 25, 907–915 (2006).

- P. So, H. Kwon, and C. Dong, “Resolution enhancement in standing-wave total internal reflection microscopy: a point-spread-function engineering approach,” J. Opt. Soc. Am. A, 18, 2833-2845 (2001).

- P. Rangarajan, V. Bhakta, and M. Christensen, "Surpassing the Diffraction Limit of Digital Imaging Systems Using Sinusoidal Illumination Patterns," in Computational Optical Sensing and Imaging, OSA Technical Digest (CD) (Optical Society of America, 2009), paper CTuC4.

- Rangarajan, P., & Papamichalis, P. (2009, November). Estimating homographies without normalization. In Image Processing (ICIP), 2009 16th IEEE International Conference on (pp. 3517-3520). IEEE.

- C. J. R. Sheppard, “Super-resolution in confocal imaging,” Optik (Stuttg.) 80, 53–54 (1988).

- Wilson, Tony, et al. "Confocal microscopy by aperture correlation." Optics letters 21.23 (1996): 1879-1881.

- C. B. Müller and J. Enderlein, “Image scanning microscopy,” Phys. Rev. Lett. 104(19), 198101 (2010)

- York, Andrew G., et al. "Resolution doubling in live, multicellular organisms via multifocal structured illumination microscopy." Nature methods 9.7 (2012): 749-754.

- Schulz, Olaf, et al. "Resolution doubling in fluorescence microscopy with confocal spinning-disk image scanning microscopy." Proceedings of the National Academy of Sciences 110.52 (2013): 21000-21005

- A. Greengard, Y. Y. Schechner, and R. Piestun, “Depth from diffracted rotation,” Opt. Lett. 31, 181–183 (2006).

- S. Quirin and R. Piestun, "Depth estimation and image recovery using broadband, incoherent illumination with engineered point spread functions," Appl. Opt.  52, A367-A376 (2013).

- M. Mazilu, J. Baumgartl, S. Kosmeier, and K. Dholakia, "Optical Eigenmodes; exploiting the quadratic nature of the energy flux and of scattering interactions," Opt. Express  19, 933-945 (2011).

- I. Leizerson, S. Lipson, and V. Sarafis, "Superresolution in far-field imaging," J. Opt. Soc. Am. A  19, 436-443 (2002).

- N. Miller, M. Dierking, and B. Duncan, "Optical sparse aperture imaging," Appl. Opt.  46, 5933-5943 (2007).

- B. Braunecker, R. Hauck, and W. Rhodes, “Pupil function replication in OTF synthesis,” Appl. Opt.  18, 44-51 (1979).

- J. Jahns, A. W. Lohmann, “OTF synthesis: low pass and band pass filtering realized by periodic replication of the pupil function,” Opt. Comm. 42, 231-236 (1982).

- P. Rangarajan, M. Christensen, and P. Milojkovic, "Parsimony in PSF engineering using patterned illumination," in Imaging and Applied Optics Postdeadline, OSA Postdeadline Paper Digest (Optical Society of America, 2013), paper CTh3C.1.